Dimensions and References–Pipe Threads, Flanges, and Pressure Conversion Factors

美国标准锥管，NPT (摘选自ANSI B1.20.1) American Standard Taper Pipe Thread, NPT (excerpt from ANSI B1.20.1)

| 公称通径 Nominal Size | 外径 Outside Diameter | 每英寸 螺纹数量 Number of Threads Per Inch | 螺纹的 斜度 Pitch of Thread | 正常手动 啮合深度 Normal Engagement By Hand | 有效螺纹 长度 Length of Effective Thread | 用于内端螺纹 的扳手装 配长度 Wrench Makeup Length for Internal Thread | 管道到消 失点的全长 Total Length: End of Pipe to Vanish Point | 外端螺纹开始 点的螺纹中径 Pitch Diameter at Beginning of External Thread | 内端螺纹开始 点的螺纹中径 Pitch Diameter at Beginning of Internal Thread | 螺纹高度 (最大) Height of Thread (Max.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | n | P | L1 | L2 | L3 | L4 | E0 | E1 | h |
| 英寸 in. | 英寸 in. |  | 英寸 in. | 英寸 in. | 英寸 in. | 英寸 in. | 英寸 in. | 英寸 in. | 英寸 in. | 英寸 in. |
| DN6 1/4 | 0.540 | 18 | 0.05556 | 0.228 | 0.4018 | 0.1667 | 0.5946 | 0.47739 | 0.49163 | 0.04444 |
| DN15 1/2 | 0.840 | 14 | 0.07143 | 0.320 | 0.5337 | 0.2143 | 0.7815 | 0.75843 | 0.77843 | 0.05714 |
| DN20 3/4 | 1.050 | 14 | 0.07143 | 0.339 | 0.5457 | 0.2143 | 0.7935 | 0.96768 | 0.98887 | 0.05714 |
| DN25 1 | 1.315 | 11-1/2 | 0.08696 | 0.400 | 0.6828 | 0.2609 | 0.9845 | 1.21363 | 1.23863 | 0.06957 |
| DN32 1-1/4 | 1.660 | 11-1/2 | 0.08696 | 0.420 | 0.7068 | 0.2609 | 1.0085 | 1.55713 | 1.58338 | 0.06957 |
| DN40 1-1/2 | 1.900 | 11-1/2 | 0.08696 | 0.420 | 0.7235 | 0.2609 | 1.0252 | 1.79609 | 1.82234 | 0.06957 |
| DN50 2 | 2.375 | 11-1/2 | 0.08696 | 0.436 | 0.7565 | 0.2609 | 1.0582 | 2.26902 | 2.29627 | 0.06957 |
| DN65 2-1/2 | 2.875 | 8 | 0.12500 | 0.682 | 1.1375 | 0.2500 | 1.5712 | 2.71953 | 2.76216 | 0.10000 |
| DN80 3 | 3.500 | 8 | 0.12500 | 0.766 | 1.2000 | 0.2500 | 1.6337 | 3.34062 | 3.38850 | 0.10000 |
| DN100 4 | 4.500 | 8 | 0.12500 | 0.844 | 1.3000 | 0.2500 | 1.7337 | 4.33438 | 4.38712 | 0.10000 |

1cm=0.397inch